Minimum Unit Type Definitions and Minimum Requirements:

Engine

- Pump:  ≥ 1000 gpm (must have current pump test)

- Water Tank:  ≥ 300 gal

- Hose:  minimum 800’ dual 3” or 800’ > 4” Hose

- Hose:  minimum 400’ of 1 ½”, 1 ¾”, or 2”

- Biannual pump test certification Inspection by YCFC&FA Organization.

- Ladders: (1) Extension, (1) Roof, (1) Attic or Combination

Truck

- Aerial ladder or platform ≥ 75’ or greater (must be certified-YCFC&FA Organization Inspection Bi Annual w/ Certification)

- Ladders:  minimum 115’, including at least (2) extension, (2) roof, (1) Attic or Combination

- Ventilation Fan (positive or negative pressure)

- Minimum of (1) one chain saw

- Minimum of (1) one rotary saw

- Biannual pump test certification Inspection by YCFC&FA Organization (If equipped with a pump).

Tanker

- Water Tank:  ≥ 1800 gal

- Quick dump (gravity or jet-assist, direct discharge from tank) Capable of dumping Left Side, Right Side and Rear

- Minimum of (1) one porta tank >  water tank capacity (Maximum Porta Tank size require 3500 Gallons)

- Biannual pump test certification Inspection by YCFC&FA Organization.

Rescue

- Must comply with the requirements for Rescue established by the Rescue Committee and/or be certified under the PA DOH/OSFC program

Heavy Rescue – *Unit Type designation only

- Must comply with the requirements for Heavy Rescue established by the Rescue Committee and/or be certified at the Vehicle & Machinery Advanced level under the PA DOH/OSFC program

- Should check both Rescue AND Heavy Rescue Unit Types

Brush

- 4 x 4 chassis

- MAXIMUM GVWR: 19,500lbs

- Pump:  ≥ 100 gpm

- Water Tank:  ≥ 150 gallons

- Minimum of two discharges (at least one 1” or larger)

- Minimum of 400’ of 1” hose

- Minimum of two (2) Indian style tanks (metal or soft side bladder)

- Minimum of two (2) forestry rakes

- UTV/ATV do not get brush designation

- Minimum of 1 Chain saw >16” bar

- Minimum of 1 Leaf blower

Attack

Repealed

Air

- Mobile air supply/SCBA cylinder refill capability, minimum five (5) 6000psi cylinder banks

Service or Utility – *Unit Radio designation only

- Any vehicle designed for fire ground support purposes, not otherwise classified, primarily intended for hauling of equipment (Examples: pickup, cargo van, utility body truck)

Car – *Unit Radio designation only

- Any vehicle designed primarily for personnel transport (Example: SUV, passenger van, sedan)

Traffic

- Meets requirements established for traffic control units by York County Fire Police Assn.

Foam

- Unit or trailer equipped to carry and deliver mass-quantities of class A or B firefighting foam

Collapse

- Specialized rescue vehicle equipped for structure collapse, trench rescue, etc., part of recognized Technical Rescue team.

Battalion

- Fire Department supervisor in a career fire department consisting of 3 or more stations.

Warden

- Appointed Forest Fire Warden recognized by PA DCNR.

Personnel Designations

- Individuals communicating via radio may be referred to using their rank, riding position, or the designation “portable”.  These are radio identifiers only, not all will be listed units in CAD.

- Only Chief Officers will have associated Units in the Cad system.  Lower-ranking officers such as Captains and Lieutenants will be logged in CAD using the “Duty Officer ##” generic unit for the station.